|  | ABN: 39 912 868 698 M: 0413 337 873 | This SWMS has been prepared and authorised by Grange Earthworks |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Date: |  | Last Review Date: | 12 May 2020 |
|  | A: CTC Industrial Estate 460 Beaudesert Road Salisbury Qld 4107 | Name: | Leigh Crawford | Next Review Date: | 12 May 2021 |
|  |  | Position: | Director |  |  |
|  | P: PO Box 669 Annerley Qld 4103 | Signature: |  |  |  |
|  | E: leigh@grangeearthworks.com.au |  |  |  |  |

| SAFE WORK METHOD STATEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION OF WORKS / ACTIVITIES UNDERTAKEN: | SWMS 001 – General Site Activities and Policies |  |  |  |  |  |  |  |  |  |  |  |  |
| PERSON RESPONSIBLE FOR IMPLEMENTATION / MONITORING / COMPLIANCE: | NAME: |  | Leigh Crawford | POSITION: |  |  |  | Operations Manager | CONTACT NO: |  |  |  | 0413 337 873 |
| APPROXIMATE START DATE: |  |  |  |  |  | APPROXIMATE COMPLETION DATE |  |  |  |  |  |  |  |
| INCLUDES THE FOLLOWING HIGH RISK CONSTRUCTION WORK: |  | Movement of powered mobile plant |  |  |  |  | Work adjacent to a road, railway, shipping lane, or other traffic corridor used by pedestrians |  |  |  |  | Work in or near water or other liquid that involves the risk of drowning |  |
|  |  | Work in or near a shaft or trench with an excavated depth of 1.5m |  |  |  |  | Demolition of a load bearing element or structure |  |  |  |  | Principal contractor policy |  |
| DETAILS OF WORKERS CONSULTED IN THE PREPARATION OF THIS SWMS: | Leigh Crawford (Director) |  |  |  |  |  | Mick Cassidy (Consultant) |  |  |  |  |  |  |
| DATE OF CONSULTATION: | 12 May 2020 |  |  |  |  |  |  |  |  |  |  |  |  |

Safe Work Method Statement submitted to the following Principal Contractor:

| COMPANY: |  |
| --- | --- |
| NAME: |  |
| SITE ADDRESS: |  |

Safe Work Method Statement reviewed by Principal Contractor:

| NAME: |  |  |  |
| --- | --- | --- | --- |
| POSITION: |  | DATE: |  |
| SIGNATURE: |  |  |  |

| PERSONAL PROTECTIVE EQUIPMENT (AS PER SWMS REQUIREMENTS) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Risk Management Process

- The likelihood of an incident occurring as a result of the hazard will first be assessed.

| Likelihood Ratings |  |  |
| --- | --- | --- |
| A | Almost Certain | The event is expected to occur in most circumstances |
| B | Likely | The event will probably occur in most circumstances |
| C | Possible | The event may occur at some time |
| D | Unlikely | The event could occur at some time |
| E | Rare | The event may only occur in exceptional circumstances |

- The consequences (if an incident did occur) will then be determined. To determine the possible consequences, a judgement on the severity of the potential outcome will be made.

| Consequence Ratings |  |  |
| --- | --- | --- |
| 1 | Insignificant | Nil injuries |
| 2 | Minor | First aid treatment, on-site release immediately contained |
| 3 | Moderate | Medical treatment, on-site release contained with outside assistance |
| 4 | Major | Extensive injuries, loss of capability, off-site release with no detrimental affects |
| 5 | Catastrophic | Death, release off-site with detrimental effect |

- The likelihood and consequences estimates will then be combined to obtain a total risk score by using the following risk priority table.

- The following legend will be used to determine the response.

| Legend |  |  |
| --- | --- | --- |
| Score |  | Action |
| E | 18 – 25 | Extreme Risk. Requires immediate attention |
| H | 10 – 17 | High Risk. Senior management attention required urgently |
| M | 6 – 9 | Moderate Risk. Follow management instructions and procedures |
| L | 1 - 5 | Low Risk. Record and review if processes change. Monitor |

- Control measures will be implemented using the following hierarchy of controls.

| TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | C | R |  | L | C | R |  |
| Pre-start discussion and planning with client / principal contractor | Unidentified hazards Unaware of site policies | C | 4 | E 18 | - A pre-work discussion will be held and Client specific induction attended with the principal contractor / client to determine: - Location of existing services including electricity and underground services; - Areas allocated for storage of materials and equipment; - Anticipated scheduling and impact of other trades at the time of the works; - Other issues to plan and allow for the safe performance of works. | D | 3 | M 9 | Project Management Supervisor All workers |
| Adequacy of Plant | Plant Failure | C | 4 | E 18 | - Type of machines and plant selected suit particular needs addressing issues such as terrain and conditions, existing services, rated capacity and characteristics. - Operator’s manuals kept / applicable load charts available at all times. - Manufacturer’s data plates and registration details in clear view. | D | 4 | M 9 | Operators |
| Assessment of work areas | Unidentified hazards | C | 3 | H 13 | - Inspection of general work areas and equipment to ensure: - Working platforms are adequate and contain edge protection where required; - Lighting (natural or other) is adequate; - Weather conditions are suitable for the task to be performed; - Tools and equipment are free of defects / in good working order. - Personal protective equipment required is in good working order. - First aid kit is readily available. | D | 3 | M 9 | All workers |

| TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | C | R |  | L | C | R |  |
| Storage of materials and housekeeping | Slips, trips falls | C | 3 | H 13 | - Equipment and plant to be delivered and stored where arranged with client / principal contractor. - Work areas to be maintained in a clear and tidy condition. - At end of each day a thorough clean up and inspection to be performed to ensure all hazards have been removed or controlled. | D | 3 | M 9 | All workers |
| Handling of materials and equipment | Manual handling injuries | C | 4 | E 18 | - Lifting aids such as cranes, trolleys, and barrows to be used wherever possible. - Activities co-ordinated and correct positioning of work platforms and materials to avoid over reaching and over stretching. Wherever possible activities to be done at waist height. - Where manual handling is required the following to be utilised: - Minimise the weight of loads where possible so as excessive loads are not carried; - Use of team lifting; - Use of good lifting techniques as follows: - A firm grip on the load - Load close to the body - Leg muscles to do the work when lifting - Smooth lift avoiding twisting or jerking - Weights that feel excessive or above workers capacity not to be lifted | D | 3 | M 9 | All workers |
| Weather Conditions | Slips / trips | C | 5 | E 22 | - Work not to be started or will cease until weather conditions become suitable. Adverse conditions include: - an increased risk of falling due to wet and slippery work surfaces - materials being caught by the wind leading to falling objects and / or an increased risk of people falling. | D | 3 | M 9 | All workers |

| TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | C | R |  | L | C | R |  |
| Working outdoors | UV Exposure Dehydration Insects, Tick & Snakes | D | 3 | M 9 | - SPF 50+ sunscreen to be applied before and regularly during work. - Use hats, sunglasses and wear UV clothing. - Regular intake of water and utilise shade available when possible. - Identify known tick or insect infested areas, adhere to vaccination requirements and regularly apply insect repellent when applicable. - Advise supervisor of any known allergic reactions, regularly inspect skin for any abnormalities and advise of any adverse reactions. | E | 3 | M 6 | All workers |
| Lighting | Slips, trips, falls Collision | C | 5 | E 22 | - Lighting to be suitable for the tasks performed. - Lighting to be provided or work to cease where natural lighting becomes inadequate. | D | 3 | M 9 | All workers |
| First Aid Provisions | Exacerbated injuries | C | 3 | H 13 | - A first aid kit will be provided for employees, which is adequate for the type of injuries which may occur. - Kit to meet statute requirements. - Kit to be kept readily accessible to work area at all times. - Workers to be familiar with location of kit and any designated first aid personnel. | D | 3 | M 9 | All workers |

| TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | C | R |  | L | C | R |  |
| Alcohol and Drugs | Personal Injury from delayed / inaccurate responses | C | 4 | E 18 | - Alcohol and illicit drugs are not to be taken onto worksites or consumed on worksites. - Employees are not to work affected by alcohol or drugs - Persons affected by alcohol or drugs will be removed from site and will face disciplinary action. - Where prescribed or over the counter medicine is taken that may affect performance (eg. drowsiness), employees are to advise Supervisor immediately. | E | 3 | M 6 | All workers |
| Fire Fighting Equipment | Burns Explosion | C | 4 | E 18 | - An adequate dry powder type extinguisher to be available at all times. - Extinguisher to be maintained according to Australian Standard and evidence available of this maintenance. - Workers trained in the correct use of extinguishers. | E | 3 | M 6 | All workers |
| Personal Protective Equipment | Struck by objects Noise Dust | C | 4 | E 18 | - Use of items of PPE where it is not practical to control risk of injury by other means. - Type 1 protective footwear to be worn at all times. - Safety helmets to be worn where there is any reasonable risk of injury due to falling objects, contact with a moving object, contact with a fixed object. - Riggers or similar gloves used where hot or rough objects are being handled. - Hearing protection (muffs or plugs) used when excessive noise is generated. - High visibility and UV rated clothing worn at all times. | D | 3 | M 9 | All workers |

| TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | C | R |  | L | C | R |  |
| Confined spaces | Explosive / contaminated atmosphere Excessive or deficient oxygen levels | C | 4 | E 18 | - Confined space entry permit gained and requirements adhered to. - Only trained, competent and authorised persons to perform work. - Initial and Entry tests of atmosphere prior to entry and where necessary atmosphere purged. Purging agent not to introduce any additional hazards. - Risk management assessment to be documented with control measure clearly defined. - All workers to be trained and clearly understand safe work procedures and emergency plan. - Emergency response plan in place and emergency rescue equipment, personnel and first aid available. - Air in confined space continually monitored to ensure: - Oxygen levels stay between 19.5% and 23.5% - Carbon Monoxide level stays less than 30ppm - Hydrogen Sulphide level stays less than 10ppm - Combustible (LEL) level stays below 5% - Signage and / or barricading to be used to restrict unauthorised access. | E | 4 | H 10 | All workers |
| Emergency preparedness | Exacerbated injury and damage | C | 3 | H 13 | - All workers to be familiar with and adhere to site emergency procedures and emergency procedure of Grange Earthworks. - In the event of an emergency Grange Earthworks Supervisor and Site Supervisor to be notified immediately. - 1 person to calm injured worker and maintain continuous verbal communication. - Emergency services contacted immediately – Dial 000 - 1 person to await emergency services at front of site - 1 person to maintain continuous communication with injured worker. | E | 3 | M 6 | Operator Project Management |

Additional Hazards / Special Precautions / Control Measures (to be completed where review may determine necessary)

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |

Applicable Legislation, Standards, Competencies, Plant / Equipment and PPE

| Plant / Equipment Used: |  | Personal Protective Equipment Used: |  | Hazardous Chemicals / Dangerous Goods Used: |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Safety Boots AS/NZS 2210.3:2000 |  |  |  |
|  |  | High visibility clothing AS/NZS 4602:1999 and AS/NZS 4399:1996 |  |  |  |
|  |  | SPF 50+ sunscreen |  |  |  |
|  |  | Safety Helmet AS/NZS 1801 (tick if required) |  |  |  |
|  |  | Gloves AS/NZS 2161:2008 (tick if required) |  |  |  |
|  |  | Eye protection AS/NZS 1337.1 (tick if required) |  |  |  |
|  |  | Ear protection AS/NZS 1270:2002 (tick if required) |  |  |  |
|  |  | Respiratory protection AS/NZS 1716:2012 (tick if required) |  |  |  |
|  |  | Fall arrest equipment AS/NZS 1891:2009 (tick if required) |  |  |  |
|  |  | Long sleeve shirt and pants |  |  |  |
|  |  | Wide brim hats |  |  |  |

| Engineering Details / Approvals | Plant and design registration for plant where required. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance Checks | Plant and equipment – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Machines – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Electrical equipment – current test and tag at 3 monthly intervals. |  |  |  |  |  |  |  |
| Relevant Legislation, Applicable Codes of Practice | Work Health and Safety Act 2011 Work Health and Safety Regulation 2011 Electrical Safety Act 2002 Electrical Safety Regulation 2013 Hazardous Manual Tasks Code of Practice 2011 How to Manage Work Health and Safety Risks Code of Practice 2011 Labelling of Workplace Hazardous Chemicals Code of Practice 2011 Managing Noise and Preventing Hearing Loss Code of Practice 2011 Managing the Risk of Falls at Workplaces Code of Practice 2011 First Aid Code of Practice 2014 Electrical Safety Code of Practice 2010 Working near overhead and underground electric lines |  | Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 Managing Risks of Plant in the Workplace Code of Practice 2013 Environmental Protection Act 1994 Environmental Protection Regulation 2008 Environmental Protection (Waste) Policy and Regulation 2000 Environmental Protection (Water) Policy 2009 Environmental Protection (Noise) Policy 2008 Environmental Protection (Air) Policy 2008 |  |  |  |  |  |
| Training / Competencies / Certificates to perform work | General Construction Induction Training Site Specific Induction Work Activity Induction Training Equipment owner manuals Safe Work Method Statements and Safe Work Procedures Training |  | Competencies / Authorities to Work required: |  |  |  |  |  |
|  |  |  |  | LE | Excavator |  | LB | Front end loader / Back Hoe |
|  |  |  |  | LS | Skid Steer |  | LZ | Dozer |
|  |  |  |  | DG | Dogger |  | LG | Grader |
|  |  |  |  | LP | Scraper |  | LR | Roller |
| Monitoring / Evaluation | Measurement and evaluation will be an ongoing process performed principally by: - Continuous monitoring by supervisor; - on site monitoring by Director, Operations Manager and Supervisor/s; - formal site safety inspections against pre-determined criteria as per Grange Earthworks HSE Management Plan ; - formal incident investigations; and - consultation with employees and contractors. Should circumstances change during the activity work will cease and the SWMS will be amended as necessary. Re-training of workers in the new SWMS will occur. |  |  |  |  |  |  |  |
| Consultation & Communication | Grange Earthworks actively consult with workers and subcontractors in the following forms: |  |  |  |  |  |  |  |
|  | - site visits by Supervisors; - tool box talks used to induct employees and subcontractors; - staff meetings; | - correspondence to subcontractors; - other forums as determined. |  |  |  |  |  |  |

Consultation, Training and Competency Register

| Declaration by Employees and Contractors We, the undersigned, acknowledge that: - this SWMS has been developed in consultation with us; and - we have been trained in the contents of this SWMS and are fully conversant with the safety procedures and precautions; and - we will work in accordance with the procedures listed in the SWMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |