|  | ABN: 39 912 868 698 M: 0413 337 873 | This SWMS has been prepared and authorised by Grange Earthworks |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Date: |  | Last Review Date: | 12 May 2020 |
|  | A: CTC Industrial Estate 460 Beaudesert Road Salisbury Qld 4107 | Name: | Leigh Crawford | Next Review Date: | 12 May 2021 |
|  |  | Position: | Director |  |  |
|  | P: PO Box 669 Annerley Qld 4103 | Signature: |  |  |  |
|  | E: leigh@grangeearthworks.com.au |  |  |  |  |

| SAFE WORK METHOD STATEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION OF WORKS / ACTIVITIES UNDERTAKEN: | SWMS 006 – Mechanical Handling and Transportation of asbestos contaminated soil and waste |  |  |  |  |  |  |  |  |  |  |  |  |
| PERSON RESPONSIBLE FOR IMPLEMENTATION / MONITORING / COMPLIANCE: | NAME: |  | Leigh Crawford | POSITION: |  |  |  | Operations Manager | CONTACT NO: |  |  |  | 0413 337 873 |
| APPROXIMATE START DATE: |  |  |  |  |  | APPROXIMATE COMPLETION DATE |  |  |  |  |  |  |  |
| INCLUDES THE FOLLOWING HIGH RISK CONSTRUCTION WORK: |  | Movement of powered mobile plant |  |  |  |  | Work adjacent to a road, railway, shipping lane, or other traffic corridor used by pedestrians |  |  |  |  | Work in or near water or other liquid that involves the risk of drowning |  |
|  |  | Work in or near a shaft or trench with an excavated depth of 1.5m |  |  |  |  | Demolition of a load bearing element or structure |  |  |  |  | Principal contractor policy |  |
| DETAILS OF WORKERS CONSULTED IN THE PREPARATION OF THIS SWMS: | Leigh Crawford (Director) |  |  |  |  |  | Mick Cassidy (Consultant) |  |  |  |  |  |  |
| DATE OF CONSULTATION: | 12 May 2020 |  |  |  |  |  |  |  |  |  |  |  |  |

Safe Work Method Statement submitted to the following Principal Contractor:

| COMPANY: |  |
| --- | --- |
| NAME: |  |
| SITE ADDRESS: |  |

Safe Work Method Statement reviewed by Principal Contractor:

| NAME: |  |  |  |
| --- | --- | --- | --- |
| POSITION: |  | DATE: |  |
| SIGNATURE: |  |  |  |

| PERSONAL PROTECTIVE EQUIPMENT (AS PER SWMS REQUIREMENTS) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Risk Management Process

- The likelihood of an incident occurring as a result of the hazard will first be assessed.

| Likelihood Ratings |  |  |
| --- | --- | --- |
| A | Almost Certain | The event is expected to occur in most circumstances |
| B | Likely | The event will probably occur in most circumstances |
| C | Possible | The event may occur at some time |
| D | Unlikely | The event could occur at some time |
| E | Rare | The event may only occur in exceptional circumstances |

- The consequences (if an incident did occur) will then be determined. To determine the possible consequences, a judgement on the severity of the potential outcome will be made.

| Consequence Ratings |  |  |
| --- | --- | --- |
| 1 | Insignificant | Nil injuries |
| 2 | Minor | First aid treatment, on-site release immediately contained |
| 3 | Moderate | Medical treatment, on-site release contained with outside assistance |
| 4 | Major | Extensive injuries, loss of capability, off-site release with no detrimental affects |
| 5 | Catastrophic | Death, release off-site with detrimental effect |

- The likelihood and consequences estimates will then be combined to obtain a total risk score by using the following risk priority table.

- The following legend will be used to determine the response.

| Legend |  |  |
| --- | --- | --- |
| Score |  | Action |
| E | 18 – 25 | Extreme Risk. Requires immediate attention |
| H | 10 – 17 | High Risk. Senior management attention required urgently |
| M | 6 – 9 | Moderate Risk. Follow management instructions and procedures |
| L | 1 - 5 | Low Risk. Record and review if processes change. Monitor |

- Control measures will be implemented using the following hierarchy of controls.

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 1. | Arrival on site | Incorrect plant / operator scheduled | C | 3 | H 13 | - At time of booking / scheduling sufficient information gathered as to determine particular needs of the site and works to be performed. - Type of plant selected to suit these particular needs addressing issues such as site terrain and conditions, existing services, rated capacity and characteristics. | E | 3 | M 6 | Project Manager Operations Manager Operator |
|  |  | Non-compliance of plant | C | 3 | H 13 | - Operator’s manual kept and applicable load charts in cabin at all times. - Manufacturer’s data plates in clear view. - Registration and roadworthy certificates available. | E | 3 | M 6 | Operations Manager Operator |
| 2. | Pre-start discussion and planning with principal contractor | Unaware of site policies | C | 3 | H 13 | - A pre-work discussion will be held and site specific induction attended with the principal contractor to discuss: - The demolition plan for the systematic demolition of the structure; - Location of existing services including electricity and underground services; - Confirmation of disconnection / isolation of services; - Areas allocated for storage of materials and equipment; - Intended procedures for removal of waste; - Anticipated scheduling and impact of other trades / public at the time of the works; - Other issues to plan and allow for the safe performance of works. - Identify persons in charge of works, first aiders and phone numbers / UHF radio channel. - Site emergency procedures including method of communication and evacuation muster points. | E | 3 | M 6 | Project Management Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 3. | Access around site | Slips, trips and falls | C | 3 | H 13 | - General access ways to be clear of hazards. - Materials / equipment not to be stored in access ways. | E | 3 | M 6 | Operator |
| 4. | Pre-work inspection / assessment – moving powered mobile plant | Plant Failure | C | 3 | H 13 | - Prior to commencing work a pre-work inspection to be performed and recorded in accordance with manufacturer’s recommendations. - Items assessed to include location of electrical hazards, site terrain, availability and condition of PPE and first aid equipment, fire extinguisher and adequacy of this SWMS. - Pre-operational inspection performed to ensure plants and truck is free from defects or faults. Inspection to include the following: - Correct tyre pressure; - Park / foot brakes; - Steering - Warning devices – horn, flashing lights, reversing beeper, brake lights - Brake and other fluid levels - Seat belts, other safeguards as per manufacturer’s recommendations. | E | 3 | M 6 | Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 5. | Establishment of enclosure and work areas | Unrestricted Entry Slips, Trips Falls Inhalation of asbestos fibres | D | 4 | H 14 | - Access to established work area for authorised workers only – all other workers to be excluded. - Fencing and / or barricade to be provided and erected around all boundaries by customer. - Entry points to work areas clearly defined with signage “Asbestos Removal” conforming with AS1319. | E | 3 | M 6 | Supervisor |
|  |  | Contamination | D | 4 | H 14 | - All non-necessary items removed from work area. | E | 3 | M 6 | Operator |
| 6. | Excavate material | Refer relevant SWMS |  |  |  | - Excavators operated in accordance with SWMS 002. |  |  |  | Operator |
|  |  | Inhalation of asbestos fibres | C | 5 | E 22 | - Only authorised personnel in work area. - Material to be moist removal and sprayed progressively with water mist as required to minimise dust generation. - Silt fences / environmental controls utilised as necessary to capture run off. - Environmental consultant engaged by principal contractor to control monitoring in accordance with Code of Practice for Safe Removal of. - Work to cease immediately if airborne fibre levels exceed 0.1 fibres/ml. - Operator to remain inside cabin with windows closed and air-conditioning if required on recirculate. - Type P1 or P2 respirator and disposable overalls to be worn. | E | 3 | M 6 | Project Management Environmental consultant All workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 7. | Placement of loaded materials | Falling objects | C | 4 | E 18 | - Excavated materials loaded into truck and relocated on site in accordance with SWMS 002. | E | 3 | M 6 | Operator |
|  |  | Environmental harm | C | 4 | E 18 | OR ALTERNATIVELY - Placement of excavated materials on site in an area designated by the principal contractor to ensure: - Contaminated material will not be in the way and will not be subject to damage. - Further potential soil contamination is mitigated. | E | 3 | M 6 | Operator |
|  |  | Inhalation of asbestos fibres | C | 5 | E 22 | - Only authorised personnel in work area. - Material to be moist removal and sprayed progressively with water mist as required to minimise dust generation. - Environmental consultant engaged by principal contractor to control monitoring in accordance with Code of Practice for Safe Removal of Asbestos. - Work to cease immediately if airborne fibre levels exceed 0.1 fibres/ml. - Truck driver to remain inside cabin with windows closed and air-conditioning if required on recirculate. | E | 3 | M 6 | Project Management Environmental consultant All workers |
|  |  | Contamination | C | 4 | E 18 | - Sand bags utilised as necessary to capture run off. | E | 3 | M 6 | Project Management Environmental consultant All workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 8. | Waste disposal and / or removal | Contamination of soil | C | 4 | E 18 | - Environmental transportation permits and approvals to be in place (DERM). - Waste transported in a suitable and approved vehicle. - Waste taken to a suitable and approved landfill site. - Waste dockets obtained to verify safe and appropriate disposal. | E | 3 | M 6 | Driver |
| 9. | Emergency preparedness | Exacerbated injury and damage | C | 4 | E 18 | - All workers to be familiar with and adhere to site emergency procedures and policies of principal contractor. - In the event of an emergency Grange Earthmoving Supervisor and Site Supervisor to be notified immediately. - 1 person to calm injured worker and maintain continuous verbal communication. - Emergency services contacted immediately – Dial 000 - 1 person to await emergency services at front of site - 1 person to maintain continuous communication with injured worker. | E | 3 | M 6 | Driver All workers |
| 10. | Leave Site | Slips / Trips | C | 3 | H 13 | - Areas left in a clean and tidy state. | E | 3 | M 6 | Operator |
|  |  | Site management unaware | C | 3 | H 13 | - Sign out procedures of principal contractor followed. - Lock and secure site if required. | E | 3 | M 6 | Project Manager Operator |
|  |  | Struck by moving objects |  |  | H 13 | - Traffic management procedures of principal contractor followed. | E | 3 | M 6 | Project Manager Operator |
|  |  | Environmental harm | C | 3 | H 13 | - Environmental wash station / gravel driveways utilised. - Ensure that all wheels are free of dirt / mud to prevent transfer to roads and other sites and to prevent the spread of Fire Ants. | E | 3 | M 6 | Project Manager Operator |

Additional Hazards / Special Precautions / Control Measures (to be completed where review may determine necessary)

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |

Applicable Legislation, Standards, Competencies, Plant / Equipment and PPE

| Plant / Equipment Used: |  | Personal Protective Equipment Used: |  | Hazardous Chemicals / Dangerous Goods Used: |  |
| --- | --- | --- | --- | --- | --- |
| Tipper Trucks and Trailers |  | Safety Boots AS/NZS 2210.3:2000 |  | Diesel |  |
| Excavators |  | High visibility clothing AS/NZS 4602:1999 and AS/NZS 4399:1996 |  | Diesel Engine Oil |  |
|  |  | SPF 50+ sunscreen |  | Hydraulic Fluid |  |
|  |  | Safety Helmet AS/NZS 1801 (tick if required) |  |  |  |
|  |  | Gloves AS/NZS 2161:2008 (tick if required) |  |  |  |
|  |  | Eye protection AS/NZS 1337.1 (tick if required) |  |  |  |
|  |  | Ear protection AS/NZS 1270:2002 (tick if required) |  |  |  |
|  |  | Respiratory protection AS/NZS 1716:2012 (tick if required) |  |  |  |
|  |  | Fall arrest equipment AS/NZS 1891:2009 (tick if required) |  |  |  |
|  |  | Long sleeve shirt and pants |  |  |  |
|  |  | Wide brim hats |  |  |  |

| Engineering Details / Approvals | Plant and design registration for plant where required. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance Checks | Plant and equipment – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Machines – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Electrical equipment – current test and tag at 3 monthly intervals. |  |  |  |  |  |  |  |
| Relevant Legislation, Applicable Codes of Practice | Work Health and Safety Act 2011 Work Health and Safety Regulation 2011 Electrical Safety Act 2002 Electrical Safety Regulation 2013 Hazardous Manual Tasks Code of Practice 2011 How to Manage Work Health and Safety Risks Code of Practice 2011 Labelling of Workplace Hazardous Chemicals Code of Practice 2011 Managing Noise and Preventing Hearing Loss Code of Practice 2011 Managing the Risk of Falls at Workplaces Code of Practice 2011 First Aid Code of Practice 2014 Electrical Safety Code of Practice 2010 Working near overhead and underground electric lines |  | Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 Managing Risks of Plant in the Workplace Code of Practice 2013 Environmental Protection Act 1994 Environmental Protection Regulation 2008 Environmental Protection (Waste) Policy and Regulation 2000 Environmental Protection (Water) Policy 2009 Environmental Protection (Noise) Policy 2008 Environmental Protection (Air) Policy 2008 |  |  |  |  |  |
| Training / Competencies / Certificates to perform work | General Construction Induction Training Site Specific Induction Work Activity Induction Training Equipment owner manuals Safe Work Method Statements and Safe Work Procedures Training |  | Competencies / Authorities to Work required: |  |  |  |  |  |
|  |  |  |  | LE | Excavator |  | LB | Front end loader / Back Hoe |
|  |  |  |  | LS | Skid Steer |  | LZ | Dozer |
|  |  |  |  | DG | Dogger |  | LG | Grader |
|  |  |  |  | LP | Scraper |  | LR | Roller |
| Monitoring / Evaluation | Measurement and evaluation will be an ongoing process performed principally by: - Continuous monitoring by supervisor; - on site monitoring by Director, Operations Manager and Supervisor/s; - formal site safety inspections against pre-determined criteria as per Grange Earthworks HSE Management Plan ; - formal incident investigations; and - consultation with employees and contractors. Should circumstances change during the activity work will cease and the SWMS will be amended as necessary. Re-training of workers in the new SWMS will occur. |  |  |  |  |  |  |  |
| Consultation & Communication | Grange Earthworks actively consult with workers and subcontractors in the following forms: |  |  |  |  |  |  |  |
|  | - site visits by Supervisors; - tool box talks used to induct employees and subcontractors; - staff meetings; | - correspondence to subcontractors; - other forums as determined. |  |  |  |  |  |  |

Consultation, Training and Competency Register

| Declaration by Employees and Contractors We, the undersigned, acknowledge that: - this SWMS has been developed in consultation with us; and - we have been trained in the contents of this SWMS and are fully conversant with the safety procedures and precautions; and - we will work in accordance with the procedures listed in the SWMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |