|  | ABN: 39 912 868 698 M: 0413 337 873 | This SWMS has been prepared and authorised by Grange Earthworks |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Date: |  | Last Review Date: | 12 May 2020 |
|  | A: CTC Industrial Estate 460 Beaudesert Road Salisbury Qld 4107 | Name: | Leigh Crawford | Next Review Date: | 12 May 2021 |
|  |  | Position: | Director |  |  |
|  | P: PO Box 669 Annerley Qld 4103 | Signature: |  |  |  |
|  | E: leigh@grangeearthworks.com.au |  |  |  |  |

| SAFE WORK METHOD STATEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION OF WORKS / ACTIVITIES UNDERTAKEN: | SWMS 007 – Loading and Unloading Plant via Road Transport |  |  |  |  |  |  |  |  |  |  |  |  |
| PERSON RESPONSIBLE FOR IMPLEMENTATION / MONITORING / COMPLIANCE: | NAME: |  | Leigh Crawford | POSITION: |  |  |  | Operations Manager | CONTACT NO: |  |  |  | 0413 337 873 |
| APPROXIMATE START DATE: |  |  |  |  |  | APPROXIMATE COMPLETION DATE |  |  |  |  |  |  |  |
| INCLUDES THE FOLLOWING HIGH RISK CONSTRUCTION WORK: |  | Movement of powered mobile plant |  |  |  |  | Work adjacent to a road, railway, shipping lane, or other traffic corridor used by pedestrians |  |  |  |  | Work in or near water or other liquid that involves the risk of drowning |  |
|  |  | Work in or near a shaft or trench with an excavated depth of 1.5m |  |  |  |  | Demolition of a load bearing element or structure |  |  |  |  | Principal contractor policy |  |
| DETAILS OF WORKERS CONSULTED IN THE PREPARATION OF THIS SWMS: | Leigh Crawford (Director) |  |  |  |  |  | Mick Cassidy (Consultant) |  |  |  |  |  |  |
| DATE OF CONSULTATION: | 12 May 2020 |  |  |  |  |  |  |  |  |  |  |  |  |

Safe Work Method Statement submitted to the following Principal Contractor:

| COMPANY: |  |
| --- | --- |
| NAME: |  |
| SITE ADDRESS: |  |

Safe Work Method Statement reviewed by Principal Contractor:

| NAME: |  |  |  |
| --- | --- | --- | --- |
| POSITION: |  | DATE: |  |
| SIGNATURE: |  |  |  |

| PERSONAL PROTECTIVE EQUIPMENT (AS PER SWMS REQUIREMENTS) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Risk Management Process

- The likelihood of an incident occurring as a result of the hazard will first be assessed.

| Likelihood Ratings |  |  |
| --- | --- | --- |
| A | Almost Certain | The event is expected to occur in most circumstances |
| B | Likely | The event will probably occur in most circumstances |
| C | Possible | The event may occur at some time |
| D | Unlikely | The event could occur at some time |
| E | Rare | The event may only occur in exceptional circumstances |

- The consequences (if an incident did occur) will then be determined. To determine the possible consequences, a judgement on the severity of the potential outcome will be made.

| Consequence Ratings |  |  |
| --- | --- | --- |
| 1 | Insignificant | Nil injuries |
| 2 | Minor | First aid treatment, on-site release immediately contained |
| 3 | Moderate | Medical treatment, on-site release contained with outside assistance |
| 4 | Major | Extensive injuries, loss of capability, off-site release with no detrimental affects |
| 5 | Catastrophic | Death, release off-site with detrimental effect |

- The likelihood and consequences estimates will then be combined to obtain a total risk score by using the following risk priority table.

- The following legend will be used to determine the response.

| Legend |  |  |
| --- | --- | --- |
| Score |  | Action |
| E | 18 – 25 | Extreme Risk. Requires immediate attention |
| H | 10 – 17 | High Risk. Senior management attention required urgently |
| M | 6 – 9 | Moderate Risk. Follow management instructions and procedures |
| L | 1 - 5 | Low Risk. Record and review if processes change. Monitor |

- Control measures will be implemented using the following hierarchy of controls.

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 1. | Arrival on site | Incorrect plant / operator scheduled | C | 2 | M 8 | - At time of booking / scheduling sufficient information gathered as to determine particular needs of the site and works to be performed. - Type of plant selected to suit these particular needs addressing issues such as site terrain and conditions, existing services, rated capacity and characteristics. - Communication between the on site supervisor and the transport / plant operator is to occur before arrival in order to establish a suitable parking location in accordance with the traffic management plan in use. | D | 2 | L 5 | Project Manager Operations Manager Operators |
|  |  | Unaware of site safety rules, hazards and instructions | C | 4 | E 18 | - Upon arrival on site the transport / plant operator is required to park the vehicle in an allocated safe parking space as arranged with the Client or Principal Contractor. When parking the operator is required to inform the site supervisor immediately. IT IS VERY IMPORTANT AT THIS STAGE THAT NOTHING IS UNLOADED until further communication has occurred with the supervisor on site and a form of induction process is undertaken. - At the time of induction the operator’s applicable qualifications, competencies, licences, SWMS / JHA’s, machine pre-start checks, service records and PPE must be available for inspection. - Any site hazards or potential hazards should be communicated during this time. - Walk / talk through the process of the work required from beginning to completion. | D | 3 | M 9 | Operations Manager Operators |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 2. | Machinery pre-start inspection | Non-compliance of plant | C | 2 | M 8 | - Evidence of Design Registration available and evidence of current Plant Registration with WHS authority clearly marked. - Operator’s manual kept in cabin at all times. - Manufacturer’s data plates in clear view. - Clear safe and established exclusion zone maintained for truck and / or trailer to unload. - Complete plant / vehicle pre-start checklist. | E | 3 | M 6 | Project Manager Operations Manager Operators |
| 3. | Lower / Attach ramps | Crushing, Pinching, Over-exertion, slips, trips, falls | C | 3 | H 13 | - Gloves must be worn when unlatching, lowering and/or attaching ramps. - Fingers are to be kept clear of pinch points around the ramps. - If ramps are to be attached consider using a second person for a team lift. Remember to lift using the legs and bend the knees. - Inspect ramps to ensure they are in good working condition. All pins used must have a safety clip on the underside to prevent any chance of unplanned vertical movement. VERY IMPORTANT TO CHECK BOTH PINS AND SAFETY CLIPS. - Ensure machinery weight does not exceed safe working load limit of the ramps. - Make sure hinges / pins are secure and in good working condition per the manufacturer’s specifications. - Inspect surrounding area for any debris, uneven ground, steps, gutters or any other hazards that may interfere. | E | 3 | M 6 | Project Management Operators Essential Workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 4. | Remove lashing (Tie downs) | Crushing, pinching, striking | C | 3 | H 13 | - Ensure parking brake or other plant isolation is engaged. Plant wheel chocks where applicable must be removed immediately before unloading only. - Remove lashings in sequence (usually front to back). - If a lever bar is required the operator must stand to the side of the bar NEVER directly in line with it. - PPE must be worn at all times including riggers gloves, eye protection, hard hat and safety boots. | E | 3 | M 6 | Operators |
| 5. | Unload Plant | Slips, trips or falls. Plant tipping or roll over. | D | 5 | E 19 | - Access the transport deck via ramps when possible or by other means using 3 points of contact at all times. - Extra care must be taken and considered in wet conditions. - Check surrounding environment again immediately prior to starting the plant making sure nothing has changed including pedestrians or other physical obstructions. Use Essential Worker in a safe place when required. If an Essential Worker is used, clear communication and a clear view of the Essential Worker must be maintained at all times. - Raise any part of the plant including attachments off the transport deck enough to not touch the deck whilst moving. - When clear and safe to do so, move plant slowly toward the ramps. - No slewing, skidding, or skewing whilst moving the plant. - Descend the plant down the ramps at a slow and controlled pace with the tracks or wheels positioned in the center of the ramps. - Park the plant in the predetermined area within the exclusion zone. | D | 3 | M 9 | Operators Essential Worker |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 6. | Moving plant on ground surface | Material damage | C | 3 | H 13 | - Some finished surfaces ie: asphalt capping on roadways, cured concrete surfaces, tiled or paved areas should always be considered before operating plant on them. Often preventative measures are put in place prior to plant operating on them. Consultation with the job supervisor is essential if there is any doubt. | D | 3 | M 9 | Project Manager Operators |
| 7. | Unloading & Re-loading equipment / attachments | Suspended loads | D | 4 | H 14 | - Unloading / loading attachments or other equipment must be carried out with care and attention. - Where attachments have an intended purpose of being engaged by a quick hitch this must be used including the safety pin at all times. - Where required an Essential Worker is to be used to assist in the engagement of attachments. - Clear communication and visibility of the Essential Worker must be maintained at all times. - The operation of the machine whilst carrying out the task must be slow and steady. | E | 3 | M6 | Operators Essential Worker |
| 8. | Leave Site | Slips / Trips | C | 3 | H 13 | - Areas left in a clean and tidy state. | E | 3 | M 6 | Operators |
|  |  | Site management unaware | C | 3 | H 13 | - Sign out procedures of principal contractor followed. - Lock and secure site if required. | E | 3 | M 6 | Project Manager Operators |
|  |  | Struck by moving objects |  |  | H 13 | - Traffic management procedures of principal contractor followed. | E | 3 | M 6 | Project Manager Operators |
|  |  | Environmental harm | C | 3 | H 13 | - Environmental wash station / gravel driveways utilised. - Ensure that all wheels are free of dirt / mud to prevent transfer to roads and other sites and to prevent the spread of Fire Ants. | E | 3 | M 6 | Project Manager Operators |

Additional Hazards / Special Precautions / Control Measures (to be completed where review may determine necessary)

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |

Applicable Legislation, Standards, Competencies, Plant / Equipment and PPE

| Plant / Equipment Used: |  | Personal Protective Equipment Used: |  | Hazardous Chemicals / Dangerous Goods Used: |  |
| --- | --- | --- | --- | --- | --- |
| Truck (Flat bed or tipper) |  | Safety Boots AS/NZS 2210.3:2000 |  | Diesel |  |
| Excavators |  | High visibility clothing AS/NZS 4602:1999 and AS/NZS 4399:1996 |  | Diesel Engine Oil |  |
|  |  | SPF 50+ sunscreen |  | Hydraulic Fluid |  |
|  |  | Safety Helmet AS/NZS 1801 (tick if required) |  |  |  |
|  |  | Gloves AS/NZS 2161:2008 (tick if required) |  |  |  |
|  |  | Eye protection AS/NZS 1337.1 (tick if required) |  |  |  |
|  |  | Ear protection AS/NZS 1270:2002 (tick if required) |  |  |  |
|  |  | Respiratory protection AS/NZS 1716:2012 (tick if required) |  |  |  |
|  |  | Fall arrest equipment AS/NZS 1891:2009 (tick if required) |  |  |  |
|  |  | Long sleeve shirt and pants |  |  |  |
|  |  | Wide brim hats |  |  |  |

| Engineering Details / Approvals | Plant and design registration for plant where required. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance Checks | Plant and equipment – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Machines – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Electrical equipment – current test and tag at 3 monthly intervals. |  |  |  |  |  |  |  |
| Relevant Legislation, Applicable Codes of Practice | Work Health and Safety Act 2011 Work Health and Safety Regulation 2011 Electrical Safety Act 2002 Electrical Safety Regulation 2013 Hazardous Manual Tasks Code of Practice 2011 How to Manage Work Health and Safety Risks Code of Practice 2011 Labelling of Workplace Hazardous Chemicals Code of Practice 2011 Managing Noise and Preventing Hearing Loss Code of Practice 2011 Managing the Risk of Falls at Workplaces Code of Practice 2011 First Aid Code of Practice 2014 Electrical Safety Code of Practice 2010 Working near overhead and underground electric lines |  | Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 Managing Risks of Plant in the Workplace Code of Practice 2013 Environmental Protection Act 1994 Environmental Protection Regulation 2008 Environmental Protection (Waste) Policy and Regulation 2000 Environmental Protection (Water) Policy 2009 Environmental Protection (Noise) Policy 2008 Environmental Protection (Air) Policy 2008 |  |  |  |  |  |
| Training / Competencies / Certificates to perform work | General Construction Induction Training Site Specific Induction Work Activity Induction Training Equipment owner manuals Safe Work Method Statements and Safe Work Procedures Training |  | Competencies / Authorities to Work required: |  |  |  |  |  |
|  |  |  |  | LE | Excavator |  | LB | Front end loader / Back Hoe |
|  |  |  |  | LS | Skid Steer |  | LZ | Dozer |
|  |  |  |  | DG | Dogger |  | LG | Grader |
|  |  |  |  | LP | Scraper |  | LR | Roller |
| Monitoring / Evaluation | Measurement and evaluation will be an ongoing process performed principally by: - Continuous monitoring by supervisor; - on site monitoring by Director, Operations Manager and Supervisor/s; - formal site safety inspections against pre-determined criteria as per Grange Earthworks HSE Management Plan ; - formal incident investigations; and - consultation with employees and contractors. Should circumstances change during the activity work will cease and the SWMS will be amended as necessary. Re-training of workers in the new SWMS will occur. |  |  |  |  |  |  |  |
| Consultation & Communication | Grange Earthworks actively consult with workers and subcontractors in the following forms: |  |  |  |  |  |  |  |
|  | - site visits by Supervisors; - tool box talks used to induct employees and subcontractors; - staff meetings; | - correspondence to subcontractors; - other forums as determined. |  |  |  |  |  |  |

Consultation, Training and Competency Register

| Declaration by Employees and Contractors We, the undersigned, acknowledge that: - this SWMS has been developed in consultation with us; and - we have been trained in the contents of this SWMS and are fully conversant with the safety procedures and precautions; and - we will work in accordance with the procedures listed in the SWMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |