|  | ABN: 39 912 868 698 M: 0413 337 873 | This SWMS has been prepared and authorised by Grange Earthworks |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Date: |  | Last Review Date: | 12 May 2020 |
|  | A: CTC Industrial Estate 460 Beaudesert Road Salisbury Qld 4107 | Name: | Leigh Crawford | Next Review Date: | 12 May 2021 |
|  |  | Position: | Director |  |  |
|  | P: PO Box 669 Annerley Qld 4103 | Signature: |  |  |  |
|  | E: leigh@grangeearthworks.com.au |  |  |  |  |

| SAFE WORK METHOD STATEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION OF WORKS / ACTIVITIES UNDERTAKEN: | SWMS 008 – Operation of skid steer loaders |  |  |  |  |  |  |  |  |  |  |  |  |
| PERSON RESPONSIBLE FOR IMPLEMENTATION / MONITORING / COMPLIANCE: | NAME: |  | Leigh Crawford | POSITION: |  |  |  | Operations Manager | CONTACT NO: |  |  |  | 0413 337 873 |
| APPROXIMATE START DATE: |  |  |  |  |  | APPROXIMATE COMPLETION DATE |  |  |  |  |  |  |  |
| INCLUDES THE FOLLOWING HIGH RISK CONSTRUCTION WORK: |  | Movement of powered mobile plant |  |  |  |  | Work adjacent to a road, railway, shipping lane, or other traffic corridor used by pedestrians |  |  |  |  | Work in or near water or other liquid that involves the risk of drowning |  |
|  |  | Work in or near a shaft or trench with an excavated depth of 1.5m |  |  |  |  | Demolition of a load bearing element or structure |  |  |  |  | Principal contractor policy |  |
| DETAILS OF WORKERS CONSULTED IN THE PREPARATION OF THIS SWMS: | Leigh Crawford (Director) |  |  |  |  |  | Mick Cassidy (Consultant) |  |  |  |  |  |  |
| DATE OF CONSULTATION: | 12 May 2020 |  |  |  |  |  |  |  |  |  |  |  |  |

Safe Work Method Statement submitted to the following Principal Contractor:

| COMPANY: |  |
| --- | --- |
| NAME: |  |
| SITE ADDRESS: |  |

Safe Work Method Statement reviewed by Principal Contractor:

| NAME: |  |  |  |
| --- | --- | --- | --- |
| POSITION: |  | DATE: |  |
| SIGNATURE: |  |  |  |

| PERSONAL PROTECTIVE EQUIPMENT (AS PER SWMS REQUIREMENTS) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Risk Management Process

- The likelihood of an incident occurring as a result of the hazard will first be assessed.

| Likelihood Ratings |  |  |
| --- | --- | --- |
| A | Almost Certain | The event is expected to occur in most circumstances |
| B | Likely | The event will probably occur in most circumstances |
| C | Possible | The event may occur at some time |
| D | Unlikely | The event could occur at some time |
| E | Rare | The event may only occur in exceptional circumstances |

- The consequences (if an incident did occur) will then be determined. To determine the possible consequences, a judgement on the severity of the potential outcome will be made.

| Consequence Ratings |  |  |
| --- | --- | --- |
| 1 | Insignificant | Nil injuries |
| 2 | Minor | First aid treatment, on-site release immediately contained |
| 3 | Moderate | Medical treatment, on-site release contained with outside assistance |
| 4 | Major | Extensive injuries, loss of capability, off-site release with no detrimental affects |
| 5 | Catastrophic | Death, release off-site with detrimental effect |

- The likelihood and consequences estimates will then be combined to obtain a total risk score by using the following risk priority table.

- The following legend will be used to determine the response.

| Legend |  |  |
| --- | --- | --- |
| Score |  | Action |
| E | 18 – 25 | Extreme Risk. Requires immediate attention |
| H | 10 – 17 | High Risk. Senior management attention required urgently |
| M | 6 – 9 | Moderate Risk. Follow management instructions and procedures |
| L | 1 - 5 | Low Risk. Record and review if processes change. Monitor |

- Control measures will be implemented using the following hierarchy of controls.

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 1. | Arrival on site | Site management unaware of works Incorrect machine / operator scheduled Non-compliance of machine | C | 3 | H 13 | - At time of booking / scheduling sufficient information gathered as to determine particular needs of the site and any lift/s to be made. - Type of machine selected to suit these particular needs addressing issues such as site terrain and conditions, existing services, rated capacity and characteristics. - Operator’s manual kept and applicable load charts in cabin at all times. - Manufacturer’s data plates in clear view. | E | 3 | M 6 | Project Manager Operations Manager Operator |
| 2. | Delivery / float / unloading of machine | Collision Crush Injuries Property damage | C | 4 | E 18 | - Clear and safe access way provided for float / trucks to unloading area. - Principal Contractor Traffic Safety Management Plan to be adhered to. - Essential Worker used to assist truck drivers to reverse. Essential Worker to always remain in driver’s vision and / or use radio communication. - Remove all persons from area before unloading plant. - Persons are not to position themselves between a truck and equipment / materials / structures. - Safety instructions of driver to be followed at all times. - Inspection of ramps / equipment prior to unloading. - Safe unloading speed to be maintained. - Detach trailer if applicable before unloading plant off truck. - Ensure safety pin securing ramps to truck is in place. - Inspection of ramps / equipment prior to unloading. - Safe unloading speed to be maintained. - If ramps cannot be seen by the operator at any time whilst unloading an Essential Worker must be used to guide the operator in the machine down the ramps. - The Essential Worker is to be in full view of the operator at all times and clear of the plant operating radius. | E | 3 | M 6 | Project Manager Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 3. | Access around site | Slips, trips and falls | C | 3 | H 13 | - General access ways to be clear of hazards. - Materials / equipment not to be stored in access ways. | E | 3 | M 6 | Operator |
| 4. | Pre-site discussion and planning with principal contractor | Multiple | C | 3 | H 13 | - A pre-work discussion will be held and site specific induction attended with the principal contractor to determine: - Location of existing services including electricity and underground services; - Areas allocated for storage of materials and equipment; - Intended location for spoil and waste; - Anticipated scheduling and impact of other trades on site at the time of the works; - Other issues to plan and allow for the safe performance of works. | E | 3 | M 6 | Project Management Operator |
| 5. | Pre-work inspection / assessment | Existing services Plant Failure Property Damage | C | 3 | H 13 | - Prior to commencing work a pre-work inspection to be performed. - Items assessed to include location of electrical hazards, site terrain, availability and condition of PPE and first aid equipment and adequacy of this SWMS. - Pre-operational inspection performed to ensure machine/s if free from defects or faults. Inspection to include the following: - Correct tyre pressure / tracks; - Park / foot brakes; - Steering - Lift / bucket controls - Warning devices – horn, flashing lights, reversing beeper, brake lights - Hydraulics and other fluid levels - Roll over protective structures - Locking pins - Seat belts, other safeguards as per manufacturer’s recommendations. | E | 3 | M 6 | Operator |
| 6. | Access in / out of plant | Slips / trips / falls | C | 3 | H 13 | - Provision of adequate non-slip ladders, footholds, steps and grab rails so as to safely access cabins. - Correct use of these safeguards by operators. | E | 3 | M 6 | Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 7. | Operation of skid steer – loading of material | Falling objects and materials Collision Overturn | C | 4 | E 18 | - Operated by suitably trained and competent persons and in accordance with manufacturer’s instructions. - Operated only from operator’s compartment whilst seated, seat belt fastened and restraint bar down. - Check the strength and adequacy of the ground – consider rises and falls, existing or recently backfilled trenches. - Operate with the bucket in the lowest possible position for stability. - Do not travel across slopes. Travel up and down with the weight uphill. - Underground services to be identified with principal contractor prior to works commencing. - A safe travel speed maintained at all times. - Plant not to be left unattended. Lower bucket to ground, disengage controls, apply the park brake, switch off engine and remove key when not in use. - Ensure plant is fitted with ROPS/FOPS complying with AS 2294. | E | 3 | M 6 | Operator |
|  |  | Falling objects and materials | C | 4 | E 18 | - Do not carry others on plant and plant only driven from operator’s seat. - Unauthorised persons kept away from the plant. Do not pass bucket or loads over others. - Area clearly marked and barricaded where necessary to make safe from other traffic and pedestrians. - Maintain a suitable exclusion zone around the plant. - Risk assessment must be done to establish a suitable exclusion zone when working at heights/above other workers and/or on slopes. - Always maintain constant contact- visual and/or verbal with Essential Workers. - Ensure plant is fitted with ROPS/FOPS complying with AS 2294. | E | 3 | M 6 | Operator |
|  |  | Existing underground services | C | 4 | E 18 | - Underground services to be identified with principal contractor prior to works commencing. | E | 3 | M 6 | Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 8. | Placement of loaded materials | Falling objects | C | 4 | E 18 | - Excavated materials loaded into truck to ensure: - Materials not to be passed over anyone / cabin of vehicle. - Gross vehicle limits not to be exceeded. | E | 3 | M 6 | Operator |
|  |  | Trench / excavation collapse | C | 4 | E 18 | ALTERNATIVELY - Placement of excavated materials on site to ensure: - General access ways remain clear; - Not to present a hazard to other contractors / where arranged with principal contractor. - Sufficient distance from existing excavations so as not to present risk of collapse. - Rescue plan in place in the event of a collapse. | E | 3 | M 6 | Operator |
| 9. | Working near existing services | Electrocution Explosion | D | 5 | E 19 | - Existing services will be identified with the principal contractor prior to commencement. - An exclusion zone of 8 metres around overhead powerlines (up to 132kV) maintained which allows for sway and sag unless: - Documentation from the power supply authority confirms the lines have been de-energised; or - A suitably qualified safety observer is available when the crane could enter the exclusion zone and power supply authority has been as well as a documented safe system of work developed. | E | 5 | H 15 | Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 10. | Inspection, Repairs and Maintenance | Plant Failure Damage to Property and Persons | D | 4 | H 14 | - Planned inspections and preventative maintenance programs for plant in accordance with manufacturer’s recommendations and relevant Australian Standards. (AS2250) - As a minimum this inspection program to include: - Daily / pre-operational inspections; - Routine weekly, monthly, quarterly and annual maintenance and independent inspections. - Inspections and maintenance to take place on level ground, with bucket lowered and machine turned off. Chock tracks if working under the machine. - Do not attempt to maintain moving parts; two or more persons should work jointly. Avoid entanglement of jewellery and clothing in moving parts. - Records kept of all repair / replacement action required and taken in the form of a log book. - Instruction manuals giving sufficient information for operation, repairs and maintenance to be available at site of operation. - Up to date log books and inspection reports also available for inspection at site of operation. | E | 4 | H 10 | Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 11. | Slinging / preparation of loads | Falling objects Machine failure / overturn | C | 4 | E 18 | - Only approved, tagged and engineered slings, chains and other lifting gear complying with relevant Australian Standards to be used. - Confirmation by competent person (dogger / operator) prior to lift that load is properly balanced, all parts of the load are secured and the load is not snagged. - Confirmation by competent person prior to lift to avoid overloading of platforms / structure. - Lifting attachments are provided with operable pins, safety latches, lifting eyes and shackles are compatible. | E | 3 | M 6 | Operator |
| 12. | Lifting of objects / materials by machine | Falling objects Machine failure / overturn | C | 4 | E 18 | - Lifting area to be barricaded or use of spotter/s to maintain an exclusion zone. - Where lane closure or similar controls required signage and barricading in accordance with AS 1742 Manual of Uniform Traffic Control Devices. - Pre-operational inspections and checks of safety devices in accordance with the operator’s manual performed by operator before use. - Lifts only performed by competent and certified operators in accordance with operations manual and manufacturer’s recommendations. - Load charts and functions and limitations to be clearly visible and followed. - Operator to have a clear view of load at all times otherwise dogger/s to be used. - The instructions of the operator and dogger to be followed at all times. - Only necessary and minimal personnel to be in lifting zone during lifting of loads. - Weather conditions to be considered prior to lifts. - Loads not to be lifted over members of the public or other trades. - Operator to remain in cabin at all times whilst load is suspended. | E | 3 | M 6 | Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 13. | Using auger attachment for bored piers | Entanglement Slips, trips, falls | C | 4 | E 18 | - Only necessary and authorised personnel to be within boring area. - Use of Essential Worker or barricade where necessary to create an exclusion zone to prevent unauthorised entry whilst in operation. - Constant visual contact between Essential Worker and operator. - Spoil removed to be placed back from edges of bored pier. - Bored pier to be protected with 900mm barricade back from leading edges or otherwise protected by customer. | E | 3 | M 6 | Operator All workers |
| 14. | Use of trench digger | Flying objects Entanglement | C | 4 | E 18 | - Other persons to be excluded from the work area – a minimum of 3 metres exclusion zone to be in place. - Use of Essential Worker or barricade where necessary to create an exclusion zone to prevent unauthorised entry whilst in operation. - Constant visual contact between Essential Worker and operator. - Ensure the trench digger attachment is in good condition and suitable for the task. | E | 3 | M 6 | Operator All workers |
| 15. | Use of Road broom | Flying objects | C | 4 | E 18 | - Other persons to be excluded from the work area – a minimum of 3 metres exclusion zone to be in place. - Use of Essential Worker or barricade where necessary to create an exclusion zone to prevent unauthorised entry whilst in operation. - Constant visual contact between Essential Worker and operator. - Ensure the road broom attachment is in good condition and suitable for the task. | E | 3 | M 6 | Operator All workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 16. | Emergency preparedness | Exacerbated injury and damage | C | 3 | H 13 | - All workers to be familiar with and adhere to site emergency procedures and emergency procedure of Grange Earthworks. - In the event of an emergency Grange Earthworks Supervisor and Site Supervisor to be notified immediately. - 1 person to calm injured worker and maintain continuous verbal communication. - Emergency services contacted immediately – Dial 000 - 1 person to await emergency services at front of site - 1 person to maintain continuous communication with injured worker. | E | 3 | M 6 | Operator Project Management |
| 17. | Confined spaces | Explosive / contaminated atmosphere Excessive or deficient oxygen levels | C | 4 | E 18 | - Confined space entry permit gained and requirements adhered to. - Only trained, competent and authorised persons to perform work. - Initial and Entry tests of atmosphere prior to entry and where necessary atmosphere purged. Purging agent not to introduce any additional hazards. - Risk management assessment to be documented with control measure clearly defined. - All workers to be trained and clearly understand safe work procedures and emergency plan. - Emergency response plan in place and emergency rescue equipment, personnel and first aid available. - Air in confined space continually monitored to ensure: - Oxygen levels stay between 19.5% and 23.5% - Carbon Monoxide level stays less than 30ppm - Hydrogen Sulphide level stays less than 10ppm - Combustible (LEL) level stays below 5% - Signage and / or barricading to be used to restrict unauthorised access. | E | 4 | H 10 | All workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 18. | Work adjacent to a roadway or traffic corridor | Collision (Vehicle and mobile plant traffic) | C | 5 | E 22 | - All workers to be trained / inducted into Principal Contractors Traffic Management Plan. - Safety instructions and directions of Traffic Control personnel to be followed. - Site rules and Site Traffic Management Plan to be adhered to at all times. - Traffic management devices including barriers, signage and bollards not to be interfered with, relocated or subjected to damage. - Defective / ineffective traffic control devices brought to the attention of the principal contractor. - Workers familiar with the basic application of the Manual for Uniform Traffic Devices Part 3 and adherence to this Part. | E | 3 | M 6 | Principal Contractor Operator |
| 19. | Work over and adjacent to water | Drowning | C | 5 | E 22 | - Workers selected to be physically capable of working in conditions and environment. - Workers trained in inherent risks and conditions. - Barriers erected as necessary to create exclusion zones and safe distance from embankments. Exclusion zones not to be breached. - Suitable length of rope and float device to be readily available on site. - Rope to be sufficient length to reach and retrieve worker in the event of failure. - All workers to be trained in and familiar with emergency contact details and site emergency procedures, which are to be readily available. | E | 3 | M 6 | Supervisor Operator All workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 20. | Work in excavations / trenches | Trench collapse Confined spaces | C | 5 | E 22 | - Trenches and excavations assessed by competent person for stability prior to any persons entering them and on daily basis. - Where trenches are 1.5 metres or deeper and need to be accessed by persons, one of the following measures will be used: - Shoring or trench shields installed according to engineering specifications; or - Trench walls benched so as they are not higher than wider – no vertical face to exceed 1.5 metres; or - Trench walls battered at an angle not exceeding 45 degrees and the bottom vertical face not exceeding 1.5 metres; or - Approval in writing from engineer that trench is safe to enter. - Shoring / shield systems inspected start of every day for their integrity / effectiveness. - Trenches and excavations backfilled as soon as practicable. Open trenches 1 metre or more deep to be barricaded. - Spoil and materials to be placed and stored back from walls of excavation. | E | 3 | M 6 | All workers |
|  |  | Slips / trips / falls | C | 4 | E 18 | - Ladder access for trenches every 9 metres. - Access points to be used. | E | 3 | M 6 | All workers |
|  |  | Ignition / Explosion | C | 4 | E 18 | - Combustion engines and other equipment which may affect air quality or pose an ignition risk not to be located in trenches. | E | 3 | M 6 | All workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 23. | Loading of plant – Moving powered mobile plant | Collision | C | 4 | E 18 | - Clear and safe access way provided for float / trucks to loading area. - Principal Contractor Traffic Safety Management Plan to be adhered to. - Essential Worker used to assist truck drivers to reverse. Essential Worker to always remain in driver’s vision and / or use radio communication. - Remove all persons from area before loading plant. - Persons are not to position themselves between a truck and equipment / materials / structures. - Safety instructions of driver to be followed at all times. | E | 3 | M 6 | Project Manager Operator |
|  |  | Plant rollover | C | 4 | E 18 | - Detach trailer if applicable before loading plant onto truck. - Ensure safety pin securing ramps to truck is in place. - Inspection of ramps / equipment prior to loading. - Safe loading speed to be maintained. | E | 3 | M 6 | Operator |
| 24. | Leave Site | Slips / Trips | C | 3 | H 13 | - Areas left in a clean and tidy state. | E | 3 | M 6 | Operator |
|  |  | Site management unaware | C | 3 | H 13 | - Sign out procedures of principal contractor followed. - Lock and secure site if required. | E | 3 | M 6 | Project Manager Operator |
|  |  | Struck by moving objects | C | 3 | H 13 | - Traffic management procedures of principal contractor followed. | E | 3 | M 6 | Project Manager Operator |
|  |  | Environmental harm | C | 3 | H 13 | - Environmental wash station / gravel driveways utilised. - Ensure that all wheels are free of dirt / mud to prevent transfer to roads and other sites and to prevent the spread of Fire Ants. | E | 3 | M 6 | Project Manager Operator |

Additional Hazards / Special Precautions / Control Measures (to be completed where review may determine necessary)

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |

Applicable Legislation, Standards, Competencies, Plant / Equipment and PPE

| Plant / Equipment Used: |  | Personal Protective Equipment Used: |  | Hazardous Chemicals / Dangerous Goods Used: |  |
| --- | --- | --- | --- | --- | --- |
| Skid Steers |  | Safety Boots AS/NZS 2210.3:2000 |  | Diesel |  |
| Trucks / Floats |  | High visibility clothing AS/NZS 4602:1999 and AS/NZS 4399:1996 |  | Diesel Engine Oil |  |
| Lifting Chains / Associated Equipment |  | SPF 50+ sunscreen |  | Hydraulic Fluid |  |
| Skid Steer attachments |  | Safety Helmet AS/NZS 1801 (tick if required) |  |  |  |
|  |  | Gloves AS/NZS 2161:2008 (tick if required) |  |  |  |
|  |  | Eye protection AS/NZS 1337.1 (tick if required) |  |  |  |
|  |  | Ear protection AS/NZS 1270:2002 (tick if required) |  |  |  |
|  |  | Respiratory protection AS/NZS 1716:2012 (tick if required) |  |  |  |
|  |  | Fall arrest equipment AS/NZS 1891:2009 (tick if required) |  |  |  |
|  |  | Long sleeve shirt and pants |  |  |  |
|  |  | Wide brim hats |  |  |  |

| Engineering Details / Approvals | Plant and design registration for plant where required. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance Checks | Plant and equipment – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Machines – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Electrical equipment – current test and tag at 3 monthly intervals. |  |  |  |  |  |  |  |
| Relevant Legislation, Applicable Codes of Practice | Work Health and Safety Act 2011 Work Health and Safety Regulation 2011 Electrical Safety Act 2002 Electrical Safety Regulation 2013 Hazardous Manual Tasks Code of Practice 2011 How to Manage Work Health and Safety Risks Code of Practice 2011 Labelling of Workplace Hazardous Chemicals Code of Practice 2011 Managing Noise and Preventing Hearing Loss Code of Practice 2011 Managing the Risk of Falls at Workplaces Code of Practice 2011 First Aid Code of Practice 2014 Electrical Safety Code of Practice 2010 Working near overhead and underground electric lines |  | Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 Managing Risks of Plant in the Workplace Code of Practice 2013 Environmental Protection Act 1994 Environmental Protection Regulation 2008 Environmental Protection (Waste) Policy and Regulation 2000 Environmental Protection (Water) Policy 2009 Environmental Protection (Noise) Policy 2008 Environmental Protection (Air) Policy 2008 |  |  |  |  |  |
| Training / Competencies / Certificates to perform work | General Construction Induction Training Site Specific Induction Work Activity Induction Training Equipment owner manuals Safe Work Method Statements and Safe Work Procedures Training |  | Competencies / Authorities to Work required: |  |  |  |  |  |
|  |  |  |  | LE | Excavator |  | LB | Front end loader / Back Hoe |
|  |  |  |  | LS | Skid Steer |  | LZ | Dozer |
|  |  |  |  | DG | Dogger |  | LG | Grader |
|  |  |  |  | LP | Scraper |  | LR | Roller |
| Monitoring / Evaluation | Measurement and evaluation will be an ongoing process performed principally by: - Continuous monitoring by supervisor; - on site monitoring by Director, Operations Manager and Supervisor/s; - formal site safety inspections against pre-determined criteria as per Grange Earthworks HSE Management Plan ; - formal incident investigations; and - consultation with employees and contractors. Should circumstances change during the activity work will cease and the SWMS will be amended as necessary. Re-training of workers in the new SWMS will occur. |  |  |  |  |  |  |  |
| Consultation & Communication | Grange Earthworks actively consult with workers and subcontractors in the following forms: |  |  |  |  |  |  |  |
|  | - site visits by Supervisors; - tool box talks used to induct employees and subcontractors; - staff meetings; | - correspondence to subcontractors; - other forums as determined. |  |  |  |  |  |  |

Consultation, Training and Competency Register

| Declaration by Employees and Contractors We, the undersigned, acknowledge that: - this SWMS has been developed in consultation with us; and - we have been trained in the contents of this SWMS and are fully conversant with the safety procedures and precautions; and - we will work in accordance with the procedures listed in the SWMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |