|  | ABN: 39 912 868 698 M: 0413 337 873 | This SWMS has been prepared and authorised by Grange Earthworks |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Date: |  | Last Review Date: | 12 May 2020 |
|  | A: CTC Industrial Estate 460 Beaudesert Road Salisbury Qld 4107 | Name: | Leigh Crawford | Next Review Date: | 12 May 2021 |
|  |  | Position: | Director |  |  |
|  | P: PO Box 669 Annerley Qld 4103 | Signature: |  |  |  |
|  | E: leigh@grangeearthworks.com.au |  |  |  |  |

| SAFE WORK METHOD STATEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION OF WORKS / ACTIVITIES UNDERTAKEN: | SWMS 009 – Operation of tipper trucks and trailers |  |  |  |  |  |  |  |  |  |  |  |  |
| PERSON RESPONSIBLE FOR IMPLEMENTATION / MONITORING / COMPLIANCE: | NAME: |  | Leigh Crawford | POSITION: |  |  |  | Operations Manager | CONTACT NO: |  |  |  | 0413 337 873 |
| APPROXIMATE START DATE: |  |  |  |  |  | APPROXIMATE COMPLETION DATE |  |  |  |  |  |  |  |
| INCLUDES THE FOLLOWING HIGH RISK CONSTRUCTION WORK: |  | Movement of powered mobile plant |  |  |  |  | Work adjacent to a road, railway, shipping lane, or other traffic corridor used by pedestrians |  |  |  |  | Work in or near water or other liquid that involves the risk of drowning |  |
|  |  | Work in or near a shaft or trench with an excavated depth of 1.5m |  |  |  |  | Demolition of a load bearing element or structure |  |  |  |  | Principal contractor policy |  |
| DETAILS OF WORKERS CONSULTED IN THE PREPARATION OF THIS SWMS: | Leigh Crawford (Director) |  |  |  |  |  | Mick Cassidy (Consultant) |  |  |  |  |  |  |
| DATE OF CONSULTATION: | 12 May 2020 |  |  |  |  |  |  |  |  |  |  |  |  |

Safe Work Method Statement submitted to the following Principal Contractor:

| COMPANY: |  |
| --- | --- |
| NAME: |  |
| SITE ADDRESS: |  |

Safe Work Method Statement reviewed by Principal Contractor:

| NAME: |  |  |  |
| --- | --- | --- | --- |
| POSITION: |  | DATE: |  |
| SIGNATURE: |  |  |  |

| PERSONAL PROTECTIVE EQUIPMENT (AS PER SWMS REQUIREMENTS) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Risk Management Process

- The likelihood of an incident occurring as a result of the hazard will first be assessed.

| Likelihood Ratings |  |  |
| --- | --- | --- |
| A | Almost Certain | The event is expected to occur in most circumstances |
| B | Likely | The event will probably occur in most circumstances |
| C | Possible | The event may occur at some time |
| D | Unlikely | The event could occur at some time |
| E | Rare | The event may only occur in exceptional circumstances |

- The consequences (if an incident did occur) will then be determined. To determine the possible consequences, a judgement on the severity of the potential outcome will be made.

| Consequence Ratings |  |  |
| --- | --- | --- |
| 1 | Insignificant | Nil injuries |
| 2 | Minor | First aid treatment, on-site release immediately contained |
| 3 | Moderate | Medical treatment, on-site release contained with outside assistance |
| 4 | Major | Extensive injuries, loss of capability, off-site release with no detrimental affects |
| 5 | Catastrophic | Death, release off-site with detrimental effect |

- The likelihood and consequences estimates will then be combined to obtain a total risk score by using the following risk priority table.

- The following legend will be used to determine the response.

| Legend |  |  |
| --- | --- | --- |
| Score |  | Action |
| E | 18 – 25 | Extreme Risk. Requires immediate attention |
| H | 10 – 17 | High Risk. Senior management attention required urgently |
| M | 6 – 9 | Moderate Risk. Follow management instructions and procedures |
| L | 1 - 5 | Low Risk. Record and review if processes change. Monitor |

- Control measures will be implemented using the following hierarchy of controls.

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 1. | Arrival on site | Incorrect plant / operator scheduled | C | 3 | H 13 | - At time of booking / scheduling sufficient information gathered as to determine particular needs of the site and works to be performed. - Type of plant selected to suit these particular needs addressing issues such as site terrain and conditions, existing services, rated capacity and characteristics. | E | 3 | M 6 | Project Manager Operations Manager Operator |
|  |  | Non-compliance of plant | C | 3 | H 13 | - Operator’s manual kept and applicable load charts in cabin at all times. - Manufacturer’s data plates in clear view. - Registration and roadworthy certificates available. | E | 3 | M 6 | Operations Manager Operator |
| 2. | Pre-site discussion and planning with principal contractor | Unaware of site policies | C | 3 | H 13 | - A pre-work discussion will be held and site specific induction attended with the principal contractor to determine: - Location of existing services including electricity and underground services; - General site terrain; - Intended location for loading. - Anticipated scheduling and impact of other trades on site at the time of the works; - Other issues to plan and allow for the safe performance of works. - This pre-work discussion must be recorded. | E | 3 | M 6 | Project Management Driver |
| 3. | Access around site | Slips, trips and falls | C | 3 | H 13 | - General access ways to be clear of hazards. - Materials / equipment not to be stored in access ways. | E | 3 | M 6 | Driver |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 4. | Pre-work inspection / assessment – Moving powered mobile plant | Plant Failure | C | 3 | H 13 | - Pre-operational inspection performed and recorded in accordance with manufacturer’s recommendations to ensure plant is free from defects or faults. Inspection to include the following: - Correct tyre pressure / tracks; - Park / foot brakes; - Steering - Warning devices – horn, flashing lights, reversing beeper, brake lights - Hydraulics and other fluid levels - Roll over protective structures - Locking pins - Seat belts, other safeguards as per manufacturer’s recommendations. | E | 3 | M 6 | Operator |
|  |  | Existing services and hazards | C | 4 | E 18 | - Prior to commencing work a pre-work inspection to be performed. - Items assessed to include location of electrical hazards, site terrain, availability and condition of PPE and first aid equipment and adequacy of this SWMS. | E | 3 | M 6 | Operator |
| 5. | Access in / out of trucks | Slips / trips / falls | C | 3 | H 13 | - Provision of adequate non-slip ladders, footholds, steps and grab rails so as to safely access cabins. - Three points of contact maintained at all times. - Face ladders / footholds to ascend and descend. - Correct use of these safeguards by operators. | E | 3 | M 6 | Driver |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 6. | Driving / operating vehicle – moving powered mobile plant | Overturn | C | 4 | E 18 | - Trucks only operated by appropriately classed license holder, experienced and competent persons. - Pre-operational inspection perform to include the following: fluid and leaks, tyre pressure, mirrors, brake, seat belt mounting and suspension, warning devices including reversing alarm and lights etc. - Safe travel speed maintained and defensive driving techniques used. - Safe distance maintained from edge of embankments and other vehicles. - Vehicles maintained in accordance with manufacturer. Evidence of maintenance to be available. - Safe vehicle mass not to be exceeded with loading of materials and equipment. - Upon shut down, park brake engaged, wheels chocked (if necessary) and keys removed from vehicle. - Adequacy of the ground considered, rises and falls, existing or recently backfilled trenches. | E | 3 | M 6 | Driver |
|  |  | Collision | C | 4 | E 18 | - Traffic rules and regulations and Site Traffic Management Plan adhered to at all times. - Area in direction of travel to be clear of people and obstructions. Use of an Essential Worker or Traffic Controller where required. | E | 3 | M 6 | Driver |
|  |  | Falling objects | C | 4 | E 18 | - Loads to be flagged where necessary and maximum overhang dimensions not to be exceeded. - Do not carry others on trucks - only in designated passenger seats. | E | 3 | M 6 | Driver |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 7. | Inspecting and covering loads | Falling Objects | C | 4 | E 18 | - All loads to be inspected prior to moving truck. - Any overhanging and/or fallen debris to be pushed back into tipper body. Where practical use plant to do this avoiding the use of manual handling. - Ensure area around truck is kept clean and free of debris. - Workers are not to climb onto the back of a truck loaded with debris. - All loads must be covered prior to moving truck. | E | 3 | M 6 | Driver |
| 8. | Repetitious tasks / Static posture | Musculoskeletal injuries | C | 3 | H 13 | - Workers stretched and warmed up prior to activities. - Sufficient rest breaks taken. - Use of job rotation where possible at regular intervals. - Ergonomic and anti-vibration equipment / tools / seating to be utilised. | D | 3 | M 9 | Driver |
| 9. | Working near an exposed energised electrical installation | Electrocution | D | 5 | E 19 | - Existing services will be identified with the principal contractor prior to commencement. - Look up to check for powerlines prior to raising tipper body. - An exclusion zone of 8 metres around overhead powerlines (up to 132kV) maintained which allows for sway and sag unless: - Documentation from the power supply authority confirms the lines have been de-energised; or - A suitably qualified safety observer is available when the truck could enter the exclusion zone as well as a documented safe system of work developed. | E | 3 | M 6 | Driver |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 10. | Inspection, Repairs and Maintenance | Plant Failure | D | 4 | H 14 | - Planned inspections and preventative maintenance programs for trucks in accordance with manufacturer’s recommendations. - As a minimum this inspection program to include; - Daily / pre-operational inspections; - Nominated manufacturer’s servicing. - Inspections and maintenance to take place on level ground, with ignition off and wheels chocked if necessary. - Isolate the plant using a tag (out of service) system. - Do not attempt to maintain moving parts; two or more persons should work jointly. Avoid entanglement of jewellery and clothing in moving parts. - Service log books kept and available. - Under no circumstances can inspections nor maintenance take place whilst tipper is in operational mode. | E | 3 | M 6 | Driver |
| 11. | Emergency preparedness | Exacerbated injury and damage | C | 4 | E 18 | - All workers to be familiar with and adhere to site emergency procedures and emergency procedure of Grange Earthworks. - In the event of an emergency Grange Earthworks Supervisor and Site Supervisor to be notified immediately. - 1 person to calm injured worker and maintain continuous verbal communication. - Emergency services contacted immediately – Dial 000 - 1 person to await emergency services at front of site - 1 person to maintain continuous communication with injured worker. | E | 3 | M 6 | Driver Project Management |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 12. | Work adjacent to a roadway or traffic corridor | Collision (Vehicle and mobile plant traffic) | C | 5 | E 22 | Grange Earthworks Pty Ltd working under principal contractor traffic management plan - All workers to be trained / inducted into Traffic Control Plan. - Safety instructions and directions of Traffic Control personnel to be followed. - Site rules and Site Traffic Management Plan to be adhered to at all times. - Traffic management devices including barriers, signage and bollards not to be interfered with, relocated or subjected to damage. - Defective / ineffective traffic control devices brought to the attention of the principal contractor - Workers familiar with the basic application of the Manual for Uniform Traffic Devices Part 3 and adherence to this Part. | E | 3 | M 6 | Principal Contractor Operator |
|  |  |  | C | 5 | E 22 | Traffic Management Plan devised by Grange Earthworks Pty Ltd - A traffic management plan to be developed by Grange Earthworks Pty Ltd and all workers trained and familiar with the plan. - Traffic Management Plan to be approved by Principal Contractor prior to work commencing. - Road work signage and devices to be erected prior to work commencing and in accordance with AS 1742.3. - Daily inspections to be carried out and documented prior to work commencing each morning. - Traffic control only by approved and accredited personnel. - Work zones to be delineated in accordance with Traffic Management Plan All workers to remain within the work zone, unless otherwise directed by Traffic Controllers - Continuous monitoring by supervisor of signage, devices and effectiveness of the Traffic Management Plan and appropriate remedial action taken as necessary. - Pre start meetings to be held to ensure all workers are aware of control measures. | E | 3 | M 6 | Supervisor Operator |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 13. | Work adjacent to operating railway and overhead equipment | Electrocution | C | 5 | E 22 | - All persons, plant, materials and operations to remain within defined work areas. - Workers on site to have undergone QR Trackside Safety Induction and Site Specific Induction Training. - Safety instructions of QR Officer/s to be observed. - 3 metre exclusion zone (persons, materials and plant) maintained from overhead services unless services have been isolated. - Where services have not been isolated: - Limiting devices installed to plant to restrict movements to no greater than a 3 metre zone. - Limiting device verified by pre-operational checks at commencement of each shift. - QR Height Limiter Verification Check completed and available as confirmation. - All persons (including operators) working under live overheads to follow conditions and procedures contained in QR WMS. | E | 3 | M 6 | QR Principal Contractor Supervisor Operator |
| 17. | Work over and adjacent to water | Drowning | C | 5 | E 22 | - Workers selected to be physically capable of working in conditions and environment. - Workers trained in inherent risks and conditions. - Barriers erected as necessary to create exclusion zones and safe distance from embankments. Exclusion zones not to be breached. - Suitable length of rope and float device to be readily available on site. - Rope to be sufficient length to reach and retrieve worker in the event of failure. - All workers to be trained in and familiar with emergency contact details and site emergency procedures, which are to be readily available. | E | 3 | M 6 | Supervisor Operator All workers |

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |
| 18. | Leave Site | Slips / Trips | C | 3 | H 13 | - Areas left in a clean and tidy state. | E | 3 | M 6 | Operator |
|  |  | Site management unaware | C | 3 | H 13 | - Sign out procedures of principal contractor followed. - Lock and secure site if required. | E | 3 | M 6 | Project Manager Operator |
|  |  | Struck by moving objects |  |  |  | - Traffic management procedures of principal contractor followed. | E | 3 | M 6 | Project Manager Operator |
|  |  | Environmental harm | C | 3 | H 13 | - Environmental wash station / gravel driveways utilised. - Ensure that all wheels are free of dirt / mud to prevent transfer to roads and other sites and to prevent the spread of Fire Ants. | E | 3 | M 6 | Project Manager Operator |

Additional Hazards / Special Precautions / Control Measures (to be completed where review may determine necessary)

| STEP | TASK / ACTIVITY | HAZARD/S | RISK CLASS (BEFORE CONTROLS) |  |  | CONTROLS AND SAFE WORK PROCEDURES | RISK CLASS (AFTER CONTROLS) |  |  | RESPONSIBLE PERSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | C | R |  | L | C | R |  |

Applicable Legislation, Standards, Competencies, Plant / Equipment and PPE

| Plant / Equipment Used: |  | Personal Protective Equipment Used: |  | Hazardous Chemicals / Dangerous Goods Used: |  |
| --- | --- | --- | --- | --- | --- |
| Tipper Trucks and Trailers |  | Safety Boots AS/NZS 2210.3:2000 |  | Diesel |  |
|  |  | High visibility clothing AS/NZS 4602:1999 and AS/NZS 4399:1996 |  | Diesel Engine Oil |  |
|  |  | SPF 50+ sunscreen |  | Hydraulic Fluid |  |
|  |  | Safety Helmet AS/NZS 1801 (tick if required) |  |  |  |
|  |  | Gloves AS/NZS 2161:2008 (tick if required) |  |  |  |
|  |  | Eye protection AS/NZS 1337.1 (tick if required) |  |  |  |
|  |  | Ear protection AS/NZS 1270:2002 (tick if required) |  |  |  |
|  |  | Respiratory protection AS/NZS 1716:2012 (tick if required) |  |  |  |
|  |  | Fall arrest equipment AS/NZS 1891:2009 (tick if required) |  |  |  |
|  |  | Long sleeve shirt and pants |  |  |  |
|  |  | Wide brim hats |  |  |  |

| Engineering Details / Approvals | Plant and design registration for plant where required. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance Checks | Plant and equipment – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Machines – visual inspection and pre-start checklist prior to use. Ongoing service and maintenance in accordance with manufacturer’s instructions and recommendations. Electrical equipment – current test and tag at 3 monthly intervals. |  |  |  |  |  |  |  |
| Relevant Legislation, Applicable Codes of Practice | Work Health and Safety Act 2011 Work Health and Safety Regulation 2011 Electrical Safety Act 2002 Electrical Safety Regulation 2013 Hazardous Manual Tasks Code of Practice 2011 How to Manage Work Health and Safety Risks Code of Practice 2011 Labelling of Workplace Hazardous Chemicals Code of Practice 2011 Managing Noise and Preventing Hearing Loss Code of Practice 2011 Managing the Risk of Falls at Workplaces Code of Practice 2011 First Aid Code of Practice 2014 Electrical Safety Code of Practice 2010 Working near overhead and underground electric lines |  | Managing Risks of Hazardous Chemicals in the Workplace Code of Practice 2013 Managing Risks of Plant in the Workplace Code of Practice 2013 Environmental Protection Act 1994 Environmental Protection Regulation 2008 Environmental Protection (Waste) Policy and Regulation 2000 Environmental Protection (Water) Policy 2009 Environmental Protection (Noise) Policy 2008 Environmental Protection (Air) Policy 2008 |  |  |  |  |  |
| Training / Competencies / Certificates to perform work | General Construction Induction Training Site Specific Induction Work Activity Induction Training Equipment owner manuals Safe Work Method Statements and Safe Work Procedures Training |  | Competencies / Authorities to Work required: |  |  |  |  |  |
|  |  |  |  | LE | Excavator |  | LB | Front end loader / Back Hoe |
|  |  |  |  | LS | Skid Steer |  | LZ | Dozer |
|  |  |  |  | DG | Dogger |  | LG | Grader |
|  |  |  |  | LP | Scraper |  | LR | Roller |
| Monitoring / Evaluation | Measurement and evaluation will be an ongoing process performed principally by: - Continuous monitoring by supervisor; - on site monitoring by Director, Operations Manager and Supervisor/s; - formal site safety inspections against pre-determined criteria as per Grange Earthworks HSE Management Plan ; - formal incident investigations; and - consultation with employees and contractors. Should circumstances change during the activity work will cease and the SWMS will be amended as necessary. Re-training of workers in the new SWMS will occur. |  |  |  |  |  |  |  |
| Consultation & Communication | Grange Earthworks actively consult with workers and subcontractors in the following forms: |  |  |  |  |  |  |  |
|  | - site visits by Supervisors; - tool box talks used to induct employees and subcontractors; - staff meetings; | - correspondence to subcontractors; - other forums as determined. |  |  |  |  |  |  |

Consultation, Training and Competency Register

| Declaration by Employees and Contractors We, the undersigned, acknowledge that: - this SWMS has been developed in consultation with us; and - we have been trained in the contents of this SWMS and are fully conversant with the safety procedures and precautions; and - we will work in accordance with the procedures listed in the SWMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |